LEAF-EB

Long-term Energy Analysis Framework - Electricity Balance

User Manual

Public release 2026.08.27

Iván Merino Rodríguez · Francisco Álvarez-Velarde · Alejandro Prada Valverde · Pablo Romojaro · Constanza Mora Inzunza · Victor J. Casas-Molina

Purpose This manual is organized around the six public examples distributed with LEAF-EB. Start with Example 01 and move forward only when you need the next capability. Every example is self-contained and commented directly in YAML with #.

Universidad Católica del Maule · CIEMAT · SCK CEN · Ghent University-imec

# Start here

LEAF-EB constructs and evaluates long-term electricity scenarios from user-defined assumptions and historical data. The assumptions may come from external planning or optimization studies, policy targets or published scenarios, or they may be defined directly in LEAF-EB. The framework constructs daily or hourly electricity balances while retaining historical temporal structure and can represent storage, commodity conversion and nuclear operating rules without optimizing the future generation mix.

## The public examples are the manual

| Example | What it adds | Resolution | Dataset |
| --- | --- | --- | --- |
| 01 basic | Deterministic balance | Daily | Daily synthetic |
| 02 Monte Carlo | Temporal uncertainty | Daily | Daily synthetic |
| 03 BESS | Battery storage | Hourly | Hourly synthetic |
| 04 commodities | CH4 storage/reconversion | Hourly | Hourly synthetic |
| 05 nuclear constant | Must-run + calendar refueling | Hourly | Hourly synthetic |
| 06 nuclear load-following | Load-following + burnup refueling | Hourly | Hourly synthetic |

Recommended workflow Do not start from an empty YAML file. Copy the public example closest to your intended analysis, change one block at a time and run LEAF-EB after each meaningful change.

## Five-minute quick start

1.  Install CPython 3.11 or newer.

2.  Create a virtual environment and install requirements.txt.

3.  Run python Runner.py --list to see the six public examples.

4.  Run python Runner.py Example_01_basic.

5.  Open Outputs/Example_01_basic/Output/Results.xlsx and Outputs/Example_01_basic/Resolved_Config.yaml.

## What # means in the input files

YAML uses # to start a comment. LEAF-EB ignores the comment; it exists only to help the reader. Public examples use comments for units, important switches and non-obvious assumptions.

simulation: monte_carlo: simulations: 5 # Small demonstration sample. seed: 12345 # Reproducible random seed. preserve_annual_targets: false # Annual energy may vary.

YAML rule Indentation is part of the file structure. Use spaces, not tabs. Keep nested blocks aligned exactly as in the examples.

# 1  Scope and boundaries

## 1.1 What LEAF-EB does

Builds future demand and generation series from dated assumptions, targets and historical temporal patterns. These inputs may define a scenario directly in LEAF-EB or reproduce and adapt assumptions from an external study.

Runs daily or hourly sequential electricity-balance calculations.

Supports deterministic and Monte Carlo simulations.

Represents BESS and selected commodity production, storage and reconversion chains.

Represents constant-output and load-following nuclear fleets, unit-level refueling and simplified fuel-use accounting.

Reports energy, peak power, short-term change, persistence, technology use and optional emissions indicators.

Exports annual nuclear information for downstream ANICCA analyses.

## 1.2 What LEAF-EB does not do

It does not optimize the future generation portfolio.

It does not calculate transmission constraints or AC/DC power flow.

It does not reproduce a full electricity market or unit-commitment problem.

Its nuclear fuel representation does not calculate isotopic depletion or assembly-level core management.

An additional generation requirement means represented generation is insufficient to meet demand. It is not, by itself, a reliability index.

## 1.3 Conceptual sequence

The calculation follows a simple sequence: historical data provide temporal patterns; the YAML defines future assumptions, dated targets and technology rules; LEAF-EB constructs the corresponding deterministic scenario series; optional Monte Carlo sampling perturbs selected series; hourly operating rules are applied when requested; the electricity balance then passes through configured flexibility stages in order.

Historical data + scenario assumptions -> future time series -> optional Monte Carlo -> hourly operating rules -> electricity balance -> BESS / commodities / reconversion -> results and diagnostics

# 2  Installation and running examples

## 2.1 Install dependencies

python -m venv .venv # Windows .venv\Scripts\activate python -m pip install --upgrade pip python -m pip install -r requirements.txt # macOS / Linux source .venv/bin/activate python -m pip install --upgrade pip python -m pip install -r requirements.txt

## 2.2 Run or list scenarios

python Runner.py --list python Runner.py Example_01_basic python Runner.py Example_06_nuclear_load_following

The .yml extension is optional. Running python Runner.py without an argument opens a numbered selector. A unique partial name can also be used; a non-unique partial name opens a choice list.

## 2.3 Where outputs go

All public examples use scenario.folder: Outputs. Each example writes to its own subfolder, for example Outputs/Example_03_bess/. This keeps generated files out of Inputs/ and makes cleanup straightforward.

# 3  Reading and editing a LEAF-EB YAML file

## 3.1 Top-level blocks

| Field / file | Meaning |
| --- | --- |
| scenario | Simulation period and output folders. |
| historical_data | Historical file, date column, resolution and unit. |
| projection | Future-series resolution and temporal variability. |
| simulation | Operational resolution, Monte Carlo and initial states. |
| demand | Demand target and dated scenario factors. |
| sources | Generation sources and their scenario assumptions. |
| commodities_input | Balance engine, BESS and commodity configuration. |
| output | Output detail, event threshold and optional emissions. |

## 3.2 Units

Dimensional public inputs carry their unit with the value. Examples include "150 MW", "4 h", "6000 MWh/day", "36.5 tHM" and "60 GWd/tHM". Historical demand and generation columns share one unit, declared once under historical_data.unit.

## 3.3 Historical-file rules

The date column name must match historical_data.date_column.

A Demand column is required.

Each standard generation source must have a matching historical column.

All demand and generation columns use the same time-step energy unit.

The public datasets are synthetic; replace them with your own data for real studies.

## 3.4 Daily projection with hourly simulation

Examples 03-06 use projection.resolution: daily and simulation.resolution: hourly. Long-term scenario values are defined at daily scale, while historical hourly profiles reconstruct within-day operation. This is useful for BESS and nuclear operating rules that cannot be interpreted adequately from daily net energy alone.

# 4  Example 01 - Basic deterministic balance

File: Inputs/Example_01_basic.yml. This is the safest starting point for a new scenario because it contains no Monte Carlo, BESS, commodity conversion or nuclear source.

python Runner.py Example_01_basic

## 4.1 What to inspect

scenario: dates and output location;

historical_data: daily synthetic dataset and shared MWh unit;

demand: final 6000 MWh/day reference;

sources: Solar, Wind, Hydro and Gas shares;

commodities_input.run_commodities: true: runs the electricity-balance engine even though no flexibility option is configured.

## 4.2 Minimal balance stage

commodities_input: # Runs the sequential electricity balance. # No storage or commodity chain is configured. run_commodities: true

Public-code improvement The 2026.08.27 public release allows the balance engine to run without Commodities_Production. This makes basic and BESS-only scenarios valid configurations rather than forcing a dummy commodity chain.

## 4.3 What to look for in Results.xlsx

Compare Demand and Generation first, then inspect surplus energy and the remaining additional generation requirement. Positive residual load is additional generation required to meet demand. Because Example 01 has no flexibility, the initial and remaining requirements are identical.

# 5  Example 02 - Monte Carlo uncertainty

File: Inputs/Example_02_monte_carlo.yml. It keeps the daily case and adds temporal stochastic sampling. The five simulations are intentionally small and exist only to demonstrate the workflow.

simulation: monte_carlo: simulations: 5 workers: 1 seed: 12345 confidence_level: 0.95 preserve_annual_targets: false technology_uncertainty: false sources: - Demand - Solar - Wind - Hydro - Gas

## 5.1 preserve_annual_targets

When false, sampled historical conditions can change annual energy as well as the timing within the year. When true, each stochastic annual series is rescaled to the corresponding deterministic annual total. The choice is methodological and should be reported explicitly.

## 5.2 Technology uncertainty is separate

technology_uncertainty controls supported distributions in data/Database.yaml. It is independent from temporal residual sampling. Keeping it false isolates uncertainty in demand and generation time series from uncertainty in technology parameters.

Do not copy the sample size Five simulations are enough to verify that the workflow runs. They are not a defensible sample size for a research result. Assess convergence for the metrics used in the study.

# 6  Example 03 - BESS

File: Inputs/Example_03_bess.yml. This example switches to hourly simulation because a 4-h battery should be demonstrated with intra-day operation rather than only daily net energy.

simulation: resolution: hourly commodities_input: BESS: Total: model: constant value: "150 MW" duration: "4 h" efficiency: 0.90 Dispatch_Order: surplus: - BESS deficit: - BESS

## 6.1 Power, energy and efficiency

The battery has 150 MW of power capacity and 4 h of duration, so the nominal stored-energy capacity is 600 MWh. efficiency is the round-trip efficiency used by the storage calculation. No hidden fallback is used for duration or efficiency.

## 6.2 Initial state

simulation.initial_conditions.bess_state_of_charge sets the starting state. The public example starts at 0.0 so the first discharge can only occur after the battery has charged from scenario surplus.

## 6.3 Interpretation

Results.xlsx reports BESS power and energy capacity, charge, battery discharge and equivalent full cycles. Compare the remaining additional generation requirement with Example 01 only as a qualitative learning exercise because Example 03 also changes the temporal resolution and source mix to create a clearer hourly storage example.

# 7  Example 04 - Methane chain

File: Inputs/Example_04_commodities.yml. It uses the same hourly electricity case and BESS as Example 03, then adds CH4 production, storage and reconversion.

Commodities_Production: CH4: share: 1.0 technology: M inputs: H2O: MED H2: AE CO2: DACG Fuel_to_Electricity: CH4: share: 1.0 technology: CCGT

## 7.1 Dispatch order

Dispatch_Order: surplus: - BESS - Commodities_Production deficit: - BESS - Fuel_to_Electricity

The YAML key deficit remains part of the current input schema. In this manual it means additional generation needed to meet demand. The terminal commodity-conversion stages appear last because the current balance engine applies them after BESS and interconnections.

## 7.2 Technology database

Electricity consumption and conversion parameters are read from data/Database.yaml. Public database references are documented in docs/DATABASE_REFERENCES.md. A user normally changes the selected technology name in the YAML rather than duplicating technology constants inside every scenario.

# 8  Examples 05-06 - Nuclear operation and refueling

These two examples are deliberately paired. They use the same historical hourly dataset, non-nuclear sources, demand, period and 300-MW deployment. Only the nuclear operating/refueling configuration changes.

| Field / file | Meaning |
| --- | --- |
| Example_05_nuclear_constant.yml | smr_300_calendar60 + must_run. Full available power while online; calendar-driven 30-day outages. |
| Example_06_nuclear_load_following.yml | smr_300_burnup60 + load_following. Output follows residual requirement within template constraints; refueling is burnup-driven. |

## 8.1 Common deployment

Nuclear_SMR300: technology_template: file: data/Nuclear_Technologies.yaml name: smr_300_calendar60 # Example 05 custom_mode: add hourly_operation: must_run capacity_additions: '2025-01-01': "300 MW"

The equivalent number of units is derived from installed capacity and the template unit_capacity. The user does not enter a reactor count separately.

## 8.2 Constant-output case

Example 05 uses the fixed-calendar template. The generic unit operates at full available power when it is not in a scheduled outage. In this public synthetic example, the first 30-day outage starts on 26 January 2027.

## 8.3 Load-following case

Nuclear_SMR300: technology_template: file: data/Nuclear_Technologies.yaml name: smr_300_burnup60 custom_mode: add hourly_operation: load_following dispatch_priority: 1 capacity_additions: '2025-01-01': "300 MW"

The burnup-driven case accumulates equivalent full-power exposure from actual generation. Because load-following produces less energy than constant-output operation, the burnup threshold is reached later. In the same synthetic case, the first 30-day outage starts on 13 July 2027.

Interpretation The date difference is not hard-coded. It emerges from the selected operating rule and refueling basis. This is why operation and fuel-cycle timing should be evaluated together.

## 8.4 ANICCA output

Nuclear runs create Output/ANICCA_Input.xlsx with Year, Installed_Power, Burnup, Load_Factor, Reactors_In and Reactors_Out. Detailed physical refueling events remain in the LEAF-EB analysis outputs.

# 9  Build your own scenario

## 9.1 Choose the nearest public example

| Field / file | Meaning |
| --- | --- |
| Only deterministic balance | Copy Example_01_basic.yml. |
| Need temporal uncertainty | Copy Example_02_monte_carlo.yml. |
| Need short-duration storage | Copy Example_03_bess.yml. |
| Need e-fuel / commodity storage | Copy Example_04_commodities.yml. |
| Need fixed-output nuclear | Copy Example_05_nuclear_constant.yml. |
| Need nuclear load-following | Copy Example_06_nuclear_load_following.yml. |

## 9.2 Change one layer at a time

1.  Copy the selected YAML to Inputs/MyScenario.yml.

2.  Replace historical_data.file and verify the date and source columns.

3.  Change scenario dates and demand target.

4.  Update source shares, capacity factors and dated values.

5.  Run the deterministic case before enabling Monte Carlo.

6.  Add BESS, commodities or nuclear only after the base case runs.

7.  Archive Resolved_Config.yaml with any published or shared results.

## 9.3 Comments are part of good public inputs

Use # comments for assumptions that another user needs to understand without opening the manual: why a switch is true or false, what a dated change represents, or which unit/boundary is intended. Avoid turning the YAML into a long essay; methodological justification belongs in the study documentation.

capacity_tolerance: "0 %" # No deviation from capacity. technology_uncertainty: false # Keep technology parameters fixed. anchor: method: auto # Let LEAF-EB resolve the anchor rule.

# 10  Outputs and interpretation

## 10.1 Core files

| Field / file | Meaning |
| --- | --- |
| Resolved_Config.yaml | Effective configuration used by the run. |
| Forecast.xlsx / Forecast.csv | Constructed demand and generation series. |
| Output/Results.xlsx | Main annual comparison tables. |
| Output/Tallies.xlsx | Analysis-level event and operating summaries. |
| Output/Diagnostics.xlsx | Warnings and diagnostic information. |
| Output/ANICCA_Input.xlsx | Annual nuclear interface when nuclear is used. |

## 10.2 Output levels

comparison is intended for compact scenario comparison; analysis adds event, duration, BESS, nuclear and other diagnostic summaries; detailed retains full time histories and can become large for hourly Monte Carlo runs.

## 10.3 Metric interpretation

LEAF-EB uses residual load with the sign convention demand minus represented supply. Negative residual load corresponds to surplus. Positive residual load corresponds to additional generation required to meet demand. The current Results.xlsx public labels retain Positive_Residual_Load terminology; interpret it using this physical meaning rather than as a reliability probability.

Peak metrics are power quantities. Energy metrics integrate the relevant part of the balance over time. Ramp metrics describe short-term change, while episode and duration summaries describe persistence.

# 11  Reproducibility and troubleshooting

## 11.1 Reproducibility checklist

Keep the exact YAML input used for the run.

Keep Resolved_Config.yaml.

Record the LEAF-EB release or Git commit.

Keep the random seed and Monte Carlo source list explicit.

Report preserve_annual_targets and technology_uncertainty.

Keep study-specific datasets outside the public repository unless their license and provenance permit redistribution.

## 11.2 Common errors

| Field / file | Meaning |
| --- | --- |
| No input matches ... | Check the filename under Inputs/ and use --list. |
| Historical column ... not found | Make the YAML source name match the historical CSV/XLSX column. |
| Unknown field ... | The public schema is strict. Check spelling and docs/INPUT_SCHEMA.md. |
| Invalid unit ... | Use an accepted dimensional form such as MW, MWh/day, h or GWd/tHM. |
| Monte Carlo does not perturb a source | Enable projection variability and list the source under monte_carlo.sources. |
| Nuclear refueling differs between examples | Check whether the template is calendar-driven or burnup-driven and whether operation is must_run or load_following. |

## 11.3 Public tests

python -m pip install -r requirements-dev.txt python -m pytest -q

The GitHub Actions workflow validates and executes all six public examples on Python 3.11 and 3.12. This provides a direct check that the manual examples remain compatible with the public code.

# Appendix A  Repository structure

LEAF-EB/ ├── Runner.py ├── Inputs/ │ ├── Example_01_basic.yml │ ├── Example_02_monte_carlo.yml │ ├── Example_03_bess.yml │ ├── Example_04_commodities.yml │ ├── Example_05_nuclear_constant.yml │ ├── Example_06_nuclear_load_following.yml │ └── data/ ├── data/ │ ├── Database.yaml │ └── Nuclear_Technologies.yaml ├── docs/ ├── scripts/ ├── src/ ├── tests/ ├── requirements.txt └── requirements-dev.txt

# Appendix B  Further technical documentation

docs/EXAMPLES.md - guided tour through the six public examples.

docs/INPUT_SCHEMA.md - current public fields and accepted units.

docs/STOCHASTIC_METHOD.md - temporal Monte Carlo method.

docs/NUCLEAR_OPERATION.md - nuclear operating and refueling model.

docs/ANALYSIS_METRICS.md - output indicators and interpretation.

docs/DATABASE_REFERENCES.md - technology parameter references.

docs/LEAF_TO_ANICCA.md - annual LEAF-EB to ANICCA interface.